Project Abstract

##### Group no. 3

Graduation project 1

CCAI-411

engine supervisor agent

| University number | Student Name |
| --- | --- |
| 2340507 | Khaled Alotaibi |
| 2342863 | Abdulhadi Alfadli |
| 2340550 | Jad Felemban |
| 2340394 | Ghassan Alrefaei |
| 2341614 | Mohammed Alnejidi |

When is preview worth acquiring? A dimensionless criterion, H/τ, tested on two plants

Preview-based control, which acts on a disturbance before that disturbance arrives, is well established in automotive thermal management. What is not established is when preview is worth acquiring at all. It costs sensing, mapping, connectivity and computation, and the literature reports its benefit case by case, with no criterion that transfers between systems.

This project proposes and tests such a criterion, governed by a single dimensionless ratio: H/τ, the available preview horizon divided by the thermal time constant of the component being protected. The hypothesis is that preview value collapses onto a single curve when plotted against H/τ, whichever physical plant produced the point. Two dissimilar plants test it: a turbocharged spark-ignition engine and an electric-vehicle battery pack. If the two do not overlap, the criterion is wrong or incomplete, and that outcome is reported as a result rather than discarded.

The engine plant is a zero-dimensional single-zone cycle model of a 3.0 L turbocharged inline-six, coupled to a three-node lumped-capacitance thermal network representing the block, the oil and the turbine housing, and calibrated against 175.5 minutes of read-only OBD-II logs recorded from the team's own vehicle across nine drives. Over 22 pooled quasi-steady operating points spanning 30–74 kPa manifold pressure, the modelled engine load agrees with the vehicle's reported load to 1.4 %, with no fitted parameters; the thesis states the limits of that agreement explicitly, since it does not by itself validate the breathing model. A reinforcement-learning supervisor is then evaluated against a production-style controller and a reactive protection policy under a fixed protocol, and the contribution of preview is isolated by re-evaluating the identical trained policy with its preview channel disabled.

Three artefacts are delivered. The criterion itself, as a design chart on which an engineer who knows a component's time constant and the horizon actually available can decide whether to buy preview before building anything. The validated simulation, the trained supervisor and the ablation table that stand behind that chart. And a working real-time application, which runs the same validated physics alongside the car on a laptop as a virtual sensor for

quantities the vehicle has no gauge for — turbine-housing temperature, charge temperature, knock margin and accumulated thermal damage — presented through a dashboard, a glanceable driver warning raised before the estimated temperature reaches the knee of the damage curve, and a post-drive review of what the model marked, written for a mechanic.

Control and observation are deliberately separated, and the separation is stated rather than implied. Inside the validated simulation the supervisor holds full authority over spark advance, mixture, boost and cooling. On the vehicle itself the interface is read-only, and structurally so: the application issues no write

commands and has no code path to the vehicle bus, and raw vehicle data are never written to disk — only the events the model marks. That boundary follows from having neither write authority over the engine computer nor any safe way to let a partially trained policy command a road car; it is reported as a scope decision rather than as an omission, and transferring the supervisor to real hardware belongs on an instrumented test bench, which is identified as future work.

Keywords: preview control; predictive thermal management; dimensionless criterion; reinforcement learning; engine thermal protection; virtual sensor; OBD-II validation.